GM Global Propulsion Systems

This document is the property of General Motors Corporation.

No rights are granted to use this document for any purpose other than the furnishing of

services or supplies to General Motors Corporation.

Engine Subassembly Fastening

SUBSYSTEM TECHNICAL SPECIFICATION

Original Date:          09/12/01

Revision Date:          01/27/17

Contact:  	David A. Wulbrecht

GMGPS – Assembly, Sealing and Fastening PMT

david.wulbrecht@gm.com

248-431-9589

1.0 INTRODUCTION

Complete, accurate, and timely communication of information is required between and among engineering organizations and disciplines at all stages of the General Motors’ Four Phase Process to insure a quality product.  As such, this document has been prepared as an aid in this communication process to provide a systematic means for keeping informed all who are involved in development, validation, and production of subassemblies for the GM Global Propulsion Systems Group.

1.1 Scope of Document

This document contains the fastening requirements for subassemblies used in GMGPS engines.  The purpose of this document is to define the fastening content, and the functional and performance requirements for the fasteners in the subassembly.

1.2 Document Management

This document contains specific requirements for fastening within the subassembly. Revisions to this document, affecting these requirements, must be approved by appropriate individuals from the component supplier, the fastener supplier and GM.

1.3 Subassembly Fastening Purpose

Integrated subassemblies, sometimes called ‘modules’, are common within the automotive industry.  Creation of these subassemblies necessarily includes manufacturing assembly operations at the tier suppliers.  Some of the subassembly content will be further processed at GM assembly facilities, particularly the tightening of captured fasteners and attachment of cables and hoses to the subassemblies.

The nature and quality of this integrated content greatly affects the downstream assembly operations at GM.  These influences include performance, appearance, noise characteristics, final assembly quality and throughput, and cost.  Integrated content will also have an impact on serviceability.

The primary purpose of the subassembly fastening activity is to ensure that GM’s expectations are met for all of the supplier-integrated fasteners and fastening interfaces.

Note:  Fasteners are to be procured only from approved suppliers on the Technically Approved Supplier List for fasteners.

2.0 APPLICABLE DOCUMENTS

2.1 Order of Precedence

A natural balancing of requirements will be recognized, and the requirements should flow down the following list in an orderly fashion.  Conflicts in this natural flow direction will be directed upward for proper resolution.  Various subsystem Product Focus Team (PDT) groups will have the responsibility of resolving these conflicts.  GMGPS will have the responsibility of balancing subsystem capabilities and requirements versus other subsystem requirements.

VEHICLE TECHNICAL SPECIFICATIONS  (VTS)



Global Propulsion Systems TECHNICAL SPECIFICATIONS



ENGINE TECHNICAL SPECIFICATIONS (ETS)



SUBASSEMBLY FASTENING SUBSYSTEM TECHNICAL SPECIFICATIONS (SSTS)



COMPONENT TECHNICAL SPECIFICATIONS (CTS)

If this document (the Subassembly Fastening Subsystem Technical Specification) conflicts with other subsystem documents, then this document shall have precedence.  If this document conflicts with GM component outline drawings then the outline drawings shall have precedence.  Any and all conflicts should be immediately brought to the attention of GMGPS for resolution.

All parts of the assembly must conform to GMW3059 Restricted and Reportable Materials.

2.2 Government Documents

2.2.1 United States

      - Federal

2.2.1.2  State

2.2.2 Canada

2.2.3 International

- ANSI Standards for Thread Forms

2.5 Supplier Documents

3.0 REQUIREMENTS

  - Subsystem Definition

3.1.1    Appearance

Fastener appearance is a result of the coating specification used to meet manufacturing and functional requirements.  Color must not be a consideration for the selection of the coating.  Refer to Table 1 in section 3.2.1.6 for details on proper coating selection.  The coating must not be damaged by handling.

    - Fastening Content

      - Functional Content

The subassembly fastening content shall perform some or all of the following functions, based on the application, as detailed in Section 3.2.1 :

- Generate clamp load

- Maintain clamp load - prevent loosening

- Locate mating components

- Control relative motion

- Control joint compression

- Resist corrosion

- Seal threads and/or the underhead surface of the fastener

- Provide electrical ground connection

- Isolate the mating component and/or attenuate noise

- Provide proper interfaces to GM assembly equipment

- Function properly when tightened to GM standard torque specifications

- Conform with serviceability requirements

- Conform with removability requirements

- Comply with GM specifications for reportable materials per GMW3059

- Conform with GM specifications for recyclability

      - Physical Content

The fastening subassembly consists of the following components:

- The fastener, consisting of a main body to carry the clamp load, a drive feature, a thread form and a point style conforming to GM and/or international standards for material and geometry

- A fastener coating for corrosion control and for control of tightening characteristics

- A fastener sealing or adhesive patch, where required

- A fastener compression limiter, where required

- A noise isolation element such as an elastomeric grommet, where required

- A retaining device to capture the fastener in a mating component, where required

- A thread insert, if required

- Tapped hole or nut

- Clamped component(s)

The fastening elements are the major interface between the subassembly and the GMGPS manufacturing assembly operations.  As such, GMGPS standard fasteners and fastener features must be used whenever the integrated fastener is driven in the GMGPS assembly plant.

The fastening elements are also a major source of proliferation in GMGPS service operations.  As such, GMGPS standard fasteners must be used whenever the integrated fastener is to be released for service usage.

This document contains two tables of standard fastener P/N’s (Table 1a and 1b in section 3.2.1.1). Table 1a fasteners are high grade bolts for usage in engine plant or supplier integrated attachments.  Table 1b fasteners are for vehicle plant attachments and legacy engines.  The fastener P/N’s in the two tables are required for M5, M6, M8 and M10 fastener usage. Use of the all of the standard parts, torque specifications and mating part dimensions in Table 1a and 1b, in conjunction with standard holes in the mating components, ensures fastening solutions that contain the necessary features to meet GM expectations. Deviations from this standard list of P/N’s must be discussed with fastening engineer and component DRE for resolution.

Use of these standard fasteners minimizes proliferation of part numbers, increases volume and results in reduced fastener cost. These parts must be procured from TASL approved suppliers. Most of the P/N’s will already have multiple global sources.

These standard fasteners have the GMW3359 coating which is suitable for most applications.

    - Environmental Exposure

The VTS, SSTS and/or CTS document will specify the exact environment for which each particular engine, subassembly and, consequently, the fastening subsystem, will be subjected.

Use of the standard finishes listed in Table 2, Section 3.2.1.6, will address all standard fluid exposure issues.

    - Interfaces

      - External Interfaces with Other Subsystems or Components

The fastening subassembly may have one or more of the following external material interfaces depending on the nature of the subassembled components:

As-formed (cast, forged or stamped) or machined aluminum – 1O max draft

As-formed (cast, forged or stamped) or machined iron or steel – 1O max draft

As-formed (cast, forged or stamped) or machined magnesium – 1O max draft

Brass fittings used as compression limiters and thread inserts

As-molded thermoset or thermoplastic composites

Elastomeric products such as grommets and seals

Elpo-coated surfaces

Use of the standard fastener finishes listed in Table 2, Section 3.2.1.6, will ensure compatibility with all anticipated material interfaces.

3.1.4.2  Internal Interfaces with Other Subsystems or Components

The fastening subassembly may have one or more of the following internal interfaces depending on the nature of the subassembled components:

Standard drilled/tapped blind hole per GMGPS specifications

Standard drilled/tapped through-hole per GMGPS specifications

Standard clearance hole for use with a fastener retainer

Standard nut per GMW specifications

    - Usable Life

The time and vehicle mileage for which the fastening subassembly is to function and perform according to its requirements is to be a minimum of ten years and 150,000 vehicle miles (240,000 Km).  Emission regulatory compliance is required to 100,000 vehicle miles (160,000 Km).  Trucks with GVW greater than 8600 Lbs require 225,000 vehicle miles (360,000 Km) for function and 120,000 vehicle miles (192,000 Km) for emissions compliance.

  - Product Characteristics

3.2.1   Performance Requirements

The fastening subassembly must meet the requirements described below.  These were previously listed in Section 3.1.2.1 A - O, Functional Content.  As such, evidence of meeting each of these requirements, or their non-applicability, must be provided through analysis, test, demonstration, or inspection. Some of these requirements do not have a generally accepted single value applicable for all engines and applications.  The Fastening PDT and/or VTS document must be used in that case to determine the requirement value for a particular application

3.2.1.1 Generate clamp load

3.2.1.1.1  General

The fastening subassembly must generate the clamp load required for the proper functioning of the joint.

A standard clampload value is used for the standard metric fastener sizes according to standard torque and coating specifications. The clampload may be a specific, engineered value only for the critical joints listed below.

| Joint Name | Tightening Specification | Clamp Load Target |
| --- | --- | --- |
| Cylinder Head to Block | Application-specific | Application-specific |
| Main Bearing Cap to Block | Application-specific | Application-specific |
| Connecting Rod Cap to Rod | Application-specific | Application-specific |
| Damper/Balancer/Pulley to Crankshaft | Application-specific | Application-specific |
| Flywheel/Flexplate to Crankshaft | Application-specific | Application-specific |
| Pulley/Sprocket to Camshaft | Application-specific | Application-specific |
| All others | Generic | Generic |

All elements of the fastening subassembly, as well as all of the mating components, must meet all functional requirements and life targets when subjected to the clamp load values defined for the joint.

The generic torque specifications and resulting clamp load values for standard fasteners with standard coatings are listed in Table 1 below. Mating component design and function must work to these standards.  Tier suppliers are to perform residual torque audits to ensure consistent torque after completion of bolt tightening process.

Table 1a: High strength bolts for usage in engine plant attachments

Note that clampload is not intended to flatten, seat or overcome spring forces of mating parts at assembly.  Parts must arrive at the GMGPS engine assembly plant in a flat, stable condition.  In case of spring forces, the spring loaded part is to be seated against the mating part with separate fixtures while the fastener is tightened.  In case of interference fit between mating parts, a separate pressing operation is required

Table 1b: Standard GM fasteners for use in Vehicle assembly plants and legacy engines

Note that clampload is not intended to flatten, seat or overcome spring forces of mating parts at assembly.  Parts must arrive at the GMGPS engine assembly plant in a flat, stable condition.  In case of spring forces, the spring loaded part is to be seated against the mating part with separate fixtures while the fastener is tightened.  In case of interference fit between mating parts, a separate pressing operation is required.

3.2.1.2  Maintain clamp load - prevent loosening

Some joint relaxation, and therefore loss of clamp load, after build is considered normal as the mating components are seated and conform to one another at the interfaces.  This is especially true if soft elements, such as paper gaskets, are used in the joint without compression limiters to carry the clamp load.  Effort should be made, however, to ensure the joint is designed to be as stiff (or ‘hard’) as possible.

All elements of the fastening subassembly must work to maintain the necessary level of clampload in the system.

- Parts must be flat and stable as received at the GMGPS engine assembly plant

- Parts must be designed to carry the maximum fastener clampload without excessive creep or other deformation in the joint leading to clampload loss detrimental to product function.

- Clamped part thickness should be 1.5 x nominal bolt diameter to achieve adequate bolt stretch for load retention.

- Parts must be designed to carry any additional clampload that may result from thermal expansion or operating loads

- Minimum surface finish requirement for the bearing surface under the bolt head:  Ra 3.0, Rz 20, Rz1max 25, Wt 20 Measurements per GMW 16676.

- In general, underhead bearing diameter and clamped column diameter must be at least as large as the standard bolts flange diameters shown in Table 1 to avoid bolt head embedment.

Adhesive patches on bolts to prevent loosening are to be avoided due to the adverse effects on assembly throughput, efficiency and quality.  Clamp load retention should be achieved with proper bolt loading, proper thread engagement, proper fastener free (or ‘grip’) length between the head and engaged threads, and avoidance of soft elements in the joint  without compression limiters.

If an adhesive patch must be applied, the following specifications and application recommendations should be followed:

| Usage | Adhesive Specification |
| --- | --- |
| M6 and Smaller | GMW14657 Grade A |
| M8 and larger | GMW14657 Grade B |
| High Temperature | GMW14657 Grade C |

3.2.1.3  Locate mating components

The fastener should not, in general, be used for precision mating of parts at the joint interface.  This can result in excessive friction (binding) during tightening and lower than desirable clamp load.  GMGPS prefers dowels to be used for that purpose rather than knurled and ground shoulders on bolts, for example.

3.2.1.4  Control relative motion

The fastener should not, in general, be used for controlling the relative motion of two parts (except for the use of clampload and friction to restrain joint motion to zero).

One common exception to this practice are bolts used to attach the exhaust manifold to the cylinder head, where thermal growth must be permitted to avoid manifold cracking.

3.2.1.5 Control joint compression

The fastener should not, in general, be used for controlling joint compression.

Compression should be controlled using

- the mating components themselves, including gasket carriers

- a separate compression limiting element in the mating components (typically when the joint contains soft elements such as gaskets

Shoulder bolts are to be avoided for general use.

3.2.1.6  Resist corrosion

Corrosion resistance is one of the functions of the bolt coatings.  Only the following coatings are to be used on fasteners in GMGPS engines:

Table 2:  Usage of GMW Finish Specifications

These coating specifications are to be clearly and completely specified on all fastener prints as listed above; no substitute specifications are acceptable.  The codes are important to the correct fastener-specific versions of GMW3179, GMW3044, and GMW15821 – do not omit the code specification on the fastener print.

Complete details about these coatings and their requirements can be found in the specifications.

3.2.1.7  Seal threads and/or the underhead surface of the fastener

The fastener may be required to seal against fluid leakage through the hole.  This situation is to be avoided.

A preapplied thread sealant patch is applied to the bolt threads for this purpose per GM specification 9885490.

Main bearing side bolts often use a sealant under the bolt head flange per GM specification 9985473.

| Usage | Sealant Specification |
| --- | --- |
| Bolt and Plug threads | 9985490 |
| Bolt flange | 9985473 |

A thread sealant application specification must be included on the fastener print per the following:

Leak performance at assembly

The total assembly must pass leak check in the engine assembly plant.  The following table contains the pressure and acceptable leak rate total for all joints.

| Cavity Pressure | Acceptable Maximum Air Leak Rate for a Given Joint |
| --- | --- |
| 21 kPa for oil | 1 sccm |
| 100 kPa for coolant | 1 sccm |

This leak rate is required to ensure proper function of the in-plant leak check for the total engine.  An integrated subassembly shall be air leak checked before leaving the supplier’s facility to ensure leak-free performance before shipping to the GM engine assembly plant.

No sealing element must be damaged or moved out of position during this testing.

The final engine assembly may also be subjected to a firing engine test in the plant.  All sealing elements must be ready for use and meet all performance specifications immediately after assembly.

        - Leak performance in service

No joint shall exhibit any sort of fluid leak, seepage, weep or other sign of fluid transfer from one cavity to another, or to the outside of the engine, during expected customer useage.

All joints will be evaluated for leak performance according to the GMW3155 component test schedule (which includes and references other GMW procedures) to ensure conformance to this requirement for each joint in the assembly.  Any joint sealing coolant will also be required to pass the GETC (Global Extended Thermal Cycle) test on dynamometer.  These tests must be run with assemblies built at the most severe limits for sealing joint tolerances – largest clearances, lowest bolt torque, poorest surface finish, etc. – to ensure meeting this performance requirement at the limits of variation.

Other component, dynamometer and vehicle tests will be run throughout the overall engine validation process.  As noted in our test plan there shall be no leak on any of those schedules; in most cases the vehicle testing is less severe than the component and dynamometer testing.

The fastening subassembly shall not have residual fluids of any kind left on its surface that could later be perceived as a leak.

3.2.1.3 Provide electrical ground connection

Where required, the fastener must provide a proper electrical ground connection.  This is typically achieved by specifying the correct finish, GMW3200.

3.2.1.4 Isolate the mating component and/or attenuate noise

Where required, the fastener may be assembled with a noise isolation element.  The component engineer, sealing engineer and noise engineer will work with the fastening engineer on the proper specifications for that element.

A GMGPS standard fastener should be integrated with the noise isolation element; avoid design of a new, unique part number for that purpose.

      - Provide proper interfaces to GMGPS assembly equipment

Standard interfaces to GMGPS assembly equipment include:

- A single fastener P/N per attachment

- All fasteners per attachment on the same plane

- Driven fasteners to be M5 or larger

- Do not use stud ended bolts

- A radius-hex head fastener must be used on all fasteners tightened at the GMGPS engine assembly plant per the chart shown below (radius permits easier socket engagement).  Internal drive fasteners shall not be used for fasteners to be driven in the GMGPS engine plant.

This feature is already integrated into the standard GMGPS fasteners listed in Table I, section 3.2.1.1.

- All fasteners driven at the GMGPS engine assembly plant shall include an anti-crossthreading point style per GMW3362.  This point style, commonly called the MATPoint®, is shown in the illustration below.

This feature is already integrated into the standard GMGPS fasteners listed in Table 1, section 3.2.1.1.

- fastener coating to assure consistent torque-tension relationship

The fastener coating determines the torque-tension characteristics of the bolt when tightened (how much clampload is generated from a given torque value).  All of the finishes recommended in Table 2, section 3.2.1.6, have controlled friction properties for correct assembly performance.

- All parts with more than one fastener that are to be tightened with a multiple spindle shall have the following minimum center to center distance between bolt hole locations:

| Bolt Size | Minimum C-C Distance |
| --- | --- |
| M6 | 45 mm |
| M8 | 45 mm |
| M10 | 59 mm |
| M12 | 87 mm |

- The socket socket diameter plus 2.0mm is the minimum straight in-line access (clearance) required in the mating part for fastener run down.  Designs shall allow adequate clearance for the use of standard catalog sockets as shown below.

- standard drilled/tapped hole specification to ensure proper joint design and stackup conditions

When the mating component will integrate a drilled/tapped hole for attachment of other components, a standard hole specification must be followed per the following table for aluminum components:

- standard mating component flange thickness to ensure proper joint design and stackup conditions

The mating component flange design must be selected to complement the standard drilled/tapped hole and the use of a GMGPS standard fastener.  Table 1 in section 3.2.1.1 shows the required flange thickness for use with the standard fastener and hole.  The mating component must be designed within this thickness range to ensure adequate thread engagement and grip length using the standard fastener and hole.

- provide fasteners captured in the subassembly as delivered to the engine assembly plant, ready to place and tighten as required by the Engine Assembly BOP.

- Fastener retention devices must not adversely affect the torque-tension relationship (increase or reduce friction on the fastener).

- The fasteners must not fall out of the assembly during shipping and handling.  Retaining devices in conjunction with the dunnage design must reliably capture the fasteners to the point of final tightening.

- The fasteners may be required to be positioned in a certain location relative to the component flange.

- The retention device needs to provide its function up until initial installation but is not expected to remain functional throughout the life of the engine (one-time use only). However, the retention device must not fall out of the joint after installation and must not be perceived by the customer as a broken part.

.Two basic technologies are approved for fastener retention:

- For mating components with thin flanges, a preapplied elastomeric bead per GM1131, Type D.  A counterbore in the mating tapped hole is required to avoid trapping excess bead material in the joint.  See standard tapped hole guidelines above for deep counterbore dimensioning.

- For mating components with thick flanges, a plastic retainer is required with a matching clearance hole design per the tables below:

3.2.1.6  Function properly when tightened to GM standard torque specifications

        - General

Fastener torque values are standardized for fasteners driven in GMGPSG engine assembly plants.  The mating components must be designed to accommodate the clamp load generated by the fasteners when tightened to those torque values.  Table 1 in section 3.2.1.1 contains the standard design clamp load values developed by the standard fasteners with standard coatings and torque specifications.

Fasteners used at sealing interfaces will have requirements for maximum bolt spacing depending on the type of seal.  Refer to the sealing technical specifications and the component technical specifications for detail on fastener spacing.

        - Threaded Inserts

Threaded inserts are used to provide fastener attachment points for components on composite intake manifolds. Inserts must be installed flush to proud of composite material.

Unless otherwise specified, the following performance requirements apply at room temperature:

Push-out/Pull-out load:	4 KN minimum

Torque to Turn:		20 NM minimum

Torque to strip threads:	18 NM minimum

        - Self-threading fasteners

Self-threading fasteners may be acceptable when used to attach components at the supplier facility that are not normally serviced. The supplier is responsible for all testing, development and design of the self-threading fastener interface. Assembly, Fastening and Sealing PMT will provide guidance for appropriate use of self-threading fasteners.  Mating component hole size will be critical to successful use of this kind of fastener.

3.2.1.7 Provide compatibility with common fastener handling equipment such as bolt feeders

Bolt feeders are commonly used at GMGPS engine assembly plants and at supplier plants.

The coatings listed in Table 2, section 3.2.1.6, are compatible with properly designed and maintained feeders.

Use of adhesive or sealant patches is detrimental to feeder performance and reliability.  Avoid the use of these patches since most bolts will be fed somewhere in the production cycle.

      - Serviceability requirements

Serviceability requirements will be determined for specific to each program application.

In general, the fastening subassembly shall not require any special tools for removal or repair of fasteners.

The TimeSert® product and installation tools will be specified for threaded hole repair.

      - Conform with removability requirements

No specific requirements exist today for removability.  In the future GM is expected to establish specifications requiring removal of fasteners without damage to the fastener or mating parts.

      - Comply with GM specifications for reportable materials per GMW3059

      - Conform with GM specifications for recyclability

      - Quality

All fasteners shall be delivered to GMGPS free of defects in functional or non-functional areas per the specifications on the fastener print.  Where a non-GMGPS fastener is used the following notes will apply:

- Surface discontinuities per ISO 6157-1

- Carburization/decarburization per GMW25

- Unspecified detail must conform to ISO 8992 Product Grade A.

- Unless otherwise specified must conform to requirements of ANSI B18.3.1M

- Dimensions must be measured prior to coating except thread pitch and major diameters

All mating components, whether loose or part of a subassembly, shall be delivered to GMGPS free of defects in functional or non-functional areas to a level specified by GMGPS Engineering.  These defects could be in the form of scratches, rough surfaces finishes, flash and burrs, or contamination.

The characteristics of mating interfaces is more critical than for most other surfaces.  The following table lists the standard surface characteristics required at flanges.  The component engineer, Assembly, Sealing and Fastening engineer and the supplier may agree on a modified value for a specific program usage.

| Min Flange Flatness (Machined & Cast or Molded) | 0.1mm / 100mm / 0.5mm Length |
| --- | --- |

This flatness recommendation should be achieved by the mating component in the free state – fastener clampload shall not be expected to flatten or straighten a component at assembly.

Flash can occur in the casting, molding, or welding process of manufacturing.  Burrs can occur during secondary manufacturing operations such as machining or insert installation.  A reasonable level shall be determined in conjunction with the Assembly, Fastening and Sealing PMT and the component design engineer.

No residual contamination shall remain on the parts that could affect the fastening subassembly.  Particularly, cutting oils, lubricating oils and assembly aids must not be left on the surfaces of fasteners, retainers, inserts, or mating components.  These fluids will lower the friction at the interfaces and adversely affect fastener retention and tightening characteristics.

3.2.2  Physical Characteristics

      - External Packaging

The fastening subsystem must fit within the envelope to be defined by the outline drawing(s) or math data of the subsystem.  However, component packaging does not relieve the requirement to use standard fasteners and holes, and to allow clearance for the use of standard tooling.

      - Clearances

Clearances between the fastening subsystem and other vehicle subsystems which could be affected by engine roll or bounce.  Generally the requirements are a minimum of 25 mm static and 10 mm at full roll.  The GM platform group may modify these requirements.

      - Mass

The fastening subsystem mass must be minimized and consistent with the levels established during the prototype phase of development.  The fastening subsystem mass must be balanced by GMGPS with other cost, noise, reliability, and durability requirements.

Mass goals do not relieve the requirement to use standard fasteners and holes, and to allow clearance for the use of standard tooling.

3.2.4  Maintenance, Service, Repair, Removal and/or Replacement

      - Maintenance

The fastening subsystem must not require maintenance during its service life.

      - Service

The fastening subsystem must not require regular scheduled service to maintain its operational performance during its service life.

3.2.4.3  Repair

The fastening subsystem is generally not considered a repairable item with the exception of repair of threaded holes in mating components.

Thread repair recommendations are usually developed with the service engineers and the mating component engineers.  The TimeSert®  thread repair inserts are the standard product to be used on GMGPS components for repair of threaded holes in cast metal components.

3.2.4.4  Removal and/or Replacement

The fastening subsystem must be capable of removal and replacement.

3.2.4.4.1  Tools and Tool Access

Standard tools shall be used in service for removing and replacing all elements of the fastening subsystem.

3.2.4.4.2 Installation and Removal Forces

Installation and removal forces must be low enough to allow service of the fastening subsystem without special tools.  Mating components such as dampers may require special tools for removal once the fastener(s) are disassembled.

3.3 Construction

3.3.1  Materials, Processes, and Parts Selection

Selection of materials, processes, and parts for the fastening subsystem must conform to applicable GM standards such as GMW25, and specifications for coatings and other features as listed above.

3.3.1.3  Parts Selection

Every effort must be made to commonize parts selected for inclusion in the fastening subsystem using, in order of preference, 1) the table of standard fasteners listed above, 2) the GM Global Fastener Catalog, and 3) existing carryover fastening elements still meeting current specifications and requirements.

All fastening system solutions must be reviewed and accepted by the program fastening engineer.

3.3.2  Identification and Markings

All fasteners shall be marked on the head with the property class marking and fastener supplier’s mark.

3.4  Documentation

3.4.1  Quality

The fastener supplier must supply a Process Failure Mode Effects Analysis (PFMEA), Process Flow and Control Plan, and Quality Control Plan identifying how Product Quality Characteristic (PQC) and Key Product Characteristics (KPC) compliance will be maintained.

3.4.2  Engineering

Documentation needed to support a Design Failure Mode Effects Analysis (DFMEA) and Validation Plan completion will be required by the Assembly, Sealing and Fastening PMT.  This group maintains  generic DFMEA’s which should be used as a basis for the specific DFMEA.  The failure rate relationship to reliability targets and confidence intervals for each validation test shall be agreed upon within GMGPS prior to start of vehicle prototype testing.  Reports on status of validation testing must be made available by the Assembly, Sealing and Fastening PDT.

Standard validation test plans for fastened joints are available from the Assembly, Sealing and Fastening PMT.  Contact the program fastening engineer for current information on DFMEA’s and joint Analysis, Development and Validation plans.

3.5  Supplier Support

3.5.1 Support During Concepting and Development

Each component supplier of the fastening subsystem must provide development and validation support for fastening subsystem concepting and development.  The support required includes but is not limited to the following items.

- The supplier(s) must provide a qualified representative to each appropriate PMT to help resolve fastening subsystem issues.  Typically, twenty-four meetings per year on an "as needed" basis will be required.

- The supplier(s) must maintain detail drawings of their respective components and maintain 3D Unigraphics (UG) capability or compatibility.  Proficiency with electronic data exchange and data management is necessary.

- The supplier(s) must participate and consult with GMGPS engineers in the design of both product and manufacturing process including tool design and process optimization.  Additionally, the supplier(s) is expected to participate in dunnage and packaging development.  The supplier(s) is encouraged to suggest new and innovative product design alternatives.

- The supplier(s) should provide support in the testing and development of fastening subsystem concepts in order to meet the performance and validation requirements.

- The supplier(s) should actively support, if needed, fastening lab testing, engine dynamometer and vehicle development testing.

- The supplier(s) must provide detailed metallurgical analysis capabilities for the fastening subsystem to allow identification of root cause failure mechanisms during development and after the end of production for defect reporting requirements.

- The supplier(s) must agree to do whatever is reasonably necessary to alter fastening subsystem specifications and/or performance to help resolve vehicle related problems.

- The supplier(s) should apply positive and irreversible corrective action for all failures during design and process evaluation and validation.

3.5.1 Support of Subsystem After Sale

The fastening subsystem shall be available to field service in the required volumes, for all design levels needs, at the required times established by GMSPO.

4.0 VALIDATION

Component and subsystem validation of the fastening subsystem is accomplished by performing the tests in the Component or Subsystem Validation Cross Reference Index, which is maintained by the Assembly, Sealing and Fastening PMT.  Additional tests may be developed to address unique failure modes identified in the DFMEA.

The specific validation plan for the subsystem, including assembly process validation, will be determined by the Assembly, Sealing and Fastening PMT, the component engineer(s) and with the supplier.

The responsibility for performance, cost, and timing of testing must be negotiated with GMGPS.

5.0 PROVISIONS FOR SHIPPING

Packaging and shipping for both engine assembly plants and service parts will be required.  Exact shipping details will be determined by the engine assembly plant engineer or GMSPO.

The fastening subsystem packaging shall have provision to prevent damage due to abrasion or breakage in transit or environmental damage in storage or in transit.

The shipping methods and dunnage design must not allow any damage to sealing elements on the fastening elements, and must not allow fluid or solid material contamination of the fastener or component surfaces.

Special care must be taken in dunnage design where captured fasteners are used; the dunnage design must:

- separate the components and fastening elements such that they are not damaged by one another in transit

- prevent the fastener from being forced out of the mating component

- aid in the retention of the fastener by trapping the fastener head against the mating flange, if possible, so that the fastener cannot fall out

6.0. NOTES

6.1 Application Specific Information

6.2 Glossary

6.2  Acronyms, Abbreviations, and Symbols

CTS		Component Technical Specifications

DFMEA		Design Failure Mode Effects Analysis

GM			General Motors

GMGPS		General Motors Global Propulsion Systems

PMT		Product Management Team

KPC		Key Product Characteristic

PAD		Product Assembly Document

PCA		Product Change Authorization

PCS		Program Content Study

PFMEA		Process Failure Mode Effects Analysis

PDT		Product Development Team

QRD		Quality, Reliability, Durability

SAE		Society of Automotive Engineers

GMSPO		Service Parts Organization

STG		Service Technology Group

TBD		To Be Determined

VDS		Vehicle Description Summary

VTS		Vehicle Technical Specifications