This document is the property of General Motors Co.  No rights are granted to use this document for any purpose other than the furnishing of services or supplies to General Motors Co.

COMPONENT TECHNICAL SPECIFICATION

ENGINE OIL PAN ASSEMBLY

For GM Part Number: 25208688

Revision Date: 26June2024

Contact:	Ikhyun Han

GMTCK – Energy Transfer HW – Lubrication PMT

Phone:	+82 32.363.6377

Email:	Ikhyun.han@gm.com

- Contents

1.	Introduction	3

2.	APPLICABLE DOCUMENTS	3

3.	PROJECT SCOPE	3

4.	CONTACT INFORMATION	3

5.	Part Numbers	4

6.	Directed-Buys AND Tier-2 Suppliers	4

7.	Material AND Appearance Requirements	4

8.	Assembly Process Control AND Production Testing Requirements	5

9.	Performance Requirements	10

10.	Analysis, Development and Validation (ADV) Testing Requirements	11

11.	supporting information	12

12.	Revision History	15

# Introduction

  - Scope of Document:

    - The purpose of this Component Technical Specification (CTS) document is to define the general and specific technical requirements for content, function and performance of the parts and assemblies described herein, and to define the tasks and documentation required of the supplier for said parts and assemblies.

    - Complete, accurate, and timely communication of information is required between and among engineering organizations and disciplines at all stages of the General Motors product development process to ensure a quality product.  As such, this document has been prepared as an aid in this communication process to provide a systematic means for keeping informed all who are involved in development, validation, and production of the General Motors (GM) parts and assemblies presented herein.

  - Document Management:

    - This document contains specific requirements for the oil pan assembly within the engine lubrication subsystem.  As such, changes that affect these requirements must be approved by the responsible individuals from the component supplier and the GM DRE prior to this document being revised.

# APPLICABLE DOCUMENTS

  - Order of Precedence:

    - This document (Component Technical Specification) is intended to be part of the GM Statement of Requirements (SOR) package.  As such, there will be several other documents within the SOR which will be referenced for the purpose of additional details or requirements.

    - This document is an APPENDIX to the main SOR document, GMW16530, which should have been included in the SOR documentation provided for quoting.  Please refer to it for the general quoting information.

    - This Component Technical Specification is intended to be consistent with other documents within the SOR.  However, in case of a conflict between documents within or referenced in this SOR, precedence is given to general SOR documents such as GMW16530, GMN10068.

  - Conflict Resolution:

    - Any and all conflicts between this document and any other applicable document should be immediately brought to the attention of the GM DRE for resolution.

# PROJECT SCOPE

  - Program Information:

### Model Year: MY28 & beyond

### Engine Configuration: 1.5L 4 cylinder

### Engine Assembly Plant Location: Mexico

### Vehicle Application: D2Ux-2

# CONTACT INFORMATION

## GM Engineering Team & Contacts:

    - Design Release Engineer (DRE): Ikhyun Han, Ikhyun.han@gm.com, +82 32.363.6744

    - Materials Information: Seongchan Pack, Seongchan.Pack@gm.com, +1 248.408.1108

    - Purchasing Information: Jacob Malone, Jacob.G.Malone@gm.com, +1 269.999.0028

    - Sealing Engineer: Ikhyun Han, Ikhyun.han@gm.com, +82 32.363.6744

    - Supplier Quality Engineer (SQE): Matthew Phinney, Matthew.phinney@gm.com, +1 248.392.5838

# Part Numbers

## Part Numbers for Assembly: * denotes DIRECTED BUY

| QTY | Part Number | Part Name | Potential Supplier |
| --- | --- | --- | --- |
| 1 | 25208688 | PAN ASM-OIL |  |
| 1 | BGH28391 | PAN-OIL (MCHG) |  |
| 12 | 11589108 | BOLT/SCREW-OIL PAN | ITW |
| 1 | 11562025 | BOLT/SCREW-OIL PAN | ITW |
| 1 | BGH31732 | BOLT/SCREW-OIL PAN | ITW |
| 2 | 11588719 | BOLT/SCREW-OIL PAN | ITW |
| 16 | 11518792 | RETAINER-M6 BOLT | Forest City Technologies |
| 2 | 3525915 | PIN-DOWEL |  |
| 1 | 55577568 | PLUG ASM-ENG OIL DRN | ITW or EPS |
| 1 | 11503302 | INSERT-METRIC COILED THREAD | Stanley Eng. Fastening |

# Directed-Buys AND Tier-2 Suppliers

## Directed Buys: The following lists components and sources that must be used, no alternate suppliers allowed without written approval of the responsible GM SQE, DRE, and Buyer.

    - None

## Tier-2 Suppliers:

## Tier 2 suppliers, which are not specified as a directed-buy, must be selected by the tier-1 supplier with the approval of the responsible GM SQE, DRE, and Buyer.

    - The goal of tier-2 selection is to optimize both cost and risk.

    - If the level of risk for any tier-2 supplier is not acceptable, the tier-1 supplier must seek out alternate sub-component suppliers or reduce risk to an acceptable level.

# Material AND Appearance Requirements

## General Notes:

## The supplier must provide a detailed material list for all subcomponent parts used in the assembly.

## If multiple raw material sources are contracted during Production, then samples of Gamma parts must be made from the material supplied from each source to ensure all materials are validated.

## All materials used for the manufacture of the assemblies must comply with GMW3059 Restricted Reportable Substances.

## The supplier must estimate the percent recyclable content according to the Recycling Design Guide document, GM502M.

## All ferrous parts must be coated to ensure there is no visual corrosion when the part is installed to the engine.

## Coatings must be approved for use by the GM SQE, DRE, and Materials Engineer to ensure safe handling practices are adhered to and the coating is compatible with engine sealants as applicable.

## If the chemistry of a raw material is changed during Production, then both the current and modified materials must complete validation testing prior to approval of the change unless otherwise agreed upon by the GM SQE, DRE, and Materials Engineer.

## Material Specifications:

    - Machined Casting Material:

      - Aluminum Die Casting per

        - GMW5M-AL-C-D-Si8Cu3Fe-F

        - GMW5M-AL-C-D-Si10Cu2Fe1-F

        - GMW5M-AL-C-D-Si9Cu3(Fe)-F

    - Suppliers must purchase GM material specifications from outside organizations such as the Global Engineering Documents/IHS Standards Store (http://global.ihs.com/).

## Appearance and Color:

    - The following appearance features and colors are acceptable guidelines for subcomponent parts of the assemblies pending final approval by the GM DRE.

      - Molded nylon is to be black in color.

      - As-processed/manufactured color for all aluminum components.

      - Media blasting is not required for this component.

      - Media blasting may be allowed as a method to eliminate sharp edges.  Use and acceptance criteria for this process must be approved by the GM SQE and DRE.  The Supplier must complete validation and PPAP to approve the use of a media-blast process.

      - As-processed/manufactured color of Zn-Ni plated carbon steel and stainless steel for metallic connectors.

      - Non-metallic connectors are to be black in color.

      - Insulation material is to be black in color.

      - Heat shield material is to be black or silver in color unless otherwise requested or agreed upon by the GM DRE.

      - Shield material to provide protection against abrasion damage is to be black in color unless otherwise requested or agreed upon by the GM DRE.

# Assembly Process Control AND Production Testing Requirements

## Assembly Process Controls

### The supplier is required to create a Process Control Plan and a Process Flow Diagram.

    - A Process Failure Mode Effects Analysis (PFMEA) must be completed before the Production machining, assembly and test equipment is purchased.

    - The supplier is responsible for resolving all issues relative to parts that are machined incorrectly, not correctly assembled, fail leak test, and/or are otherwise defective.

    - The supplier must complete and submit a 5-Phase report to the GM DRE to address all issues involving a defective part.  See Appendix D, Section 2 for further detail.

    - The supplier must replace or repair all defective parts, in accordance with GM requirements, within 5 business days of identifying the issue and at no cost to GM.

    - Error proofing is required to ensure correct subcomponent parts are present and installed properly.

    - Installation of components that require force (i.e. seals, sensors with o-rings, fittings, plugs, dowel pins) must be installed with a load transducer such that the force can be digitally recorded and tracked to the assembly serial number.

    - Automated process controls must be used to ensure acceptable part quality for all manufacturing processes such as, but not limited to, welding, brazing, heat-staking, orbital peening, and gluing processes.

    - The assemblies must be shipped directly to the engine assembly plant as complete assemblies consisting of all subcomponent parts.

    - Assemblies must be packaged appropriately to ensure no damage occurs during the shipping and handling process.

    - All control plans must document the sampling rate of parts (e.g. 2 parts/day, 5 parts/shift).

    - Master gages must be identified and calibrated according to a schedule agreed upon by the GM SQE and DRE.

    - All data collected during Production operations must be plotted with reference to the upper and lower control limits.

      - A graph of the data must be posted at or near the Production operating station.

      - A data summary must be provided to the GM SQE and DRE upon request.

## Leak Test Requirements:

## 100% of the assemblies must complete the leak test throughout the entire program; during the experimental, prototype, and production design phases.

## All parts must complete and pass the air leak test prior to shipment to GM.

## Castings and machined components may be leak checked prior to assembly at a different leak rate than specified.  However, a leak test must be completed for the final assembly in accordance with GM specifications.

## The leak test must be capable of detecting leaks between mating surfaces.

## The supplier must maintain records for the leak test data plotted with reference to the test specifications and can be tracked by oil pan serial number.

### The air leak test is intended to ensure parts are properly assembled.

#### It is not acceptable for parts to develop a leak during the life of the product, which includes engine plant hot or cold test.

#### The supplier is responsible for investigating and implementing corrective actions if any parts develop a leak during the life of the product, which includes engine plant hot or cold test.

#### Water dunk testing must be used on 100% of parts shipped up to production launch (minimum of 300 parts). Water temperature should be heated to 65-80C (if possible).

#### Dunk testing should be used on an audit basis and to detect leakage location after production launch or successful completion of engine hot testing.

## Cleanliness Requirements and Guidelines:

    - This section applies to all subcomponent parts as delivered for assembly in addition to complete assemblies.

      - The machined casting must be cleaned using an Ultrasonic washing process.

    - All parts delivered to GM must be clean and dry with no chips, oil, corrosion products, residual abrasive materials, process fluids, cutting fluids, release agents, soap-scum, detergent residues, and/or any other substance that may adversely affect function, appearance, safe handling, and/or the engine assembly process.

      - This requirement is referred to as the Level 1 cleanliness specification.

    - Debris on finished parts must be regularly monitored and controlled using statistical methods, upper control limits, and procedures approved by GM Engineering.  The procedures approved by GM must be detailed in the Supplier Process Control Plan.

      - This requirement is referred to as the Level 2 cleanliness specification.

    - The mass of debris present on any individual assembly must not exceed 10 milligrams (mg) when measured per test method GMW16037.

      - This specification applies to the final assembly as delivered from the Tier 1 Supplier.

      - This requirement is referred to as the Level 3 cleanliness specification.

    - The sampling frequency and quantity must be approved by the GM SQE and DRE and documented in the Supplier Process Control Plan.

      - A “Run Chart” must be provided to the GM SQE and DRE each month.

      - If the mass of debris contamination exceeds the upper control limit (UCL), a report in accordance with the 5-Phase Problem Resolution guidelines must be written as detailed per Section 2 of Appendix D.  Additionally, the batch of parts must be rejected unless otherwise approved by the GM SQE.

    - The following file attachments provide “best practice” guidelines for sediment reduction process controls, a test method to quantify foreign material, and an example “run chart”.

    - Dunnage (shipping containers) can be a significant factor affecting the cleanliness of parts.

      - Dunnage containers are not expendable and must be reused.

      - GM is responsible for designing the returnable dunnage.

      - The supplier is encouraged to participate in the design process and propose design concepts.

      - The supplier is responsible for cleaning the dunnage.

      - The GM Engine Assembly Plant will not accept shipments of parts that are contaminated with debris from the dunnage containers.  In this case, the cleanliness issue must be resolved by the supplier.

  - RTV Surface Preparation Requirements:

    - Part must be clean and dry with no chips, oil, residue, process fluids, cutting fluids, release agents, soap-scum, detergent residues, etc. in any area where there is to be RTV sealant contact (bead apply location as well as squeeze-out areas).

    - Wash using a safe, amine free, odorless, fast drying solvent that leaves no residue with heated water (110-150°F wash and rinse water).

    - RTV seal surfaces where RTV will be applied by the supplier must be plasma treated within 30 minutes prior to RTV application.

      - Plasma treatment must be done at a rate of less than 75 mm/second with the nozzle height located at 4 mm above the surface.

    - RTV seal surfaces where RTV will be applied by the supplier must have the surface contact angle measured after the plasma treatment application on an audit basis (not 100% check).

      - The frequency of the measurements must occur on 2 parts created at the beginning, middle and end of each shift (6 total samples).

      - Each sealing surface must be measure at a minimum of 5 locations. The contact angle measurements of each location must be less than 40 degrees.

      - The contact angle measurement should be done using a surface analyzer meter similar to Brighton Technologies Group SA3001 TSF or SA3001 TLI.

        - BTG Labs Contact: Paul McLean, pmclean@btglabs.com, (513) 478-5630

    - Wet sealant (RTV & anaerobic) application must be applied with equipment conforming to the sealant supplier’s requirements.  It must include a positive-displacement dispensing pump and error detection for missing or incomplete sealant application.

    - Surfaces intended to be sealed with RTV must pass RTV adhesion testing as defined by GMW17696.

    - The RTV sealant must approved by the GM Assembly Sealing and Fastening DRE.  The current approved material type is 9986495 (also known as Three Bond 1217H).

  - Repairs

    - There are no repair procedures approved for this part as of this writing.

    - If a repair process is approved during the program, all repair and rework processes must be documented and approved by the GM SQE and DRE.  Additionally, repaired or re-worked parts must be permanently marked via a method approved by the GM SQE and DRE and the part serial number must be documented for future reference.

    - All non-conforming parts are to be contained in a controlled area to avoid possible use of the parts.  Disposition of subcomponents and assemblies must be managed by a supervisor and not line-side workers.  Locked boxes or quarantined areas are requested.

    - Any part that fails to meet the acceptance criteria must be quarantined and clearly marked to ensure the part is not shipped to an engine assembly plant.

    - The supplier is responsible for investigating and resolving all incidents of defective assemblies (i.e. fails leak test, incorrectly machined or assembled incorrectly).   All defective parts must be replaced or repaired within 5 business days at no cost to GM.

  - Dimensional Requirements

    - Pre-Production parts must be measured to verify drawing dimensional tolerances are met.  The measurement results must be documented and provided to the GM SQE and DRE by the time the parts are delivered to GM.

      - Follow the PPAP (Production Part Approval Process) for both Prototype and Production part approval.

    - The Process Capability Procedure is attached below.

    - All data collected for the PQC’s, as applicable per drawing specifications, must be plotted against the upper and lower tolerance limits and submitted to the GM DRE at least once per month for review.

    - All PQC’s must be tracked with a “Run Chart” at a frequency agreed upon by the GM SQE and DRE.  The “Run Charts” must be submitted to the GM SQE and DRE at least once per month for review.

    - No dimensions are required to conform to KPC.

    - No dimensions are required to conform to the PQC, 6σ Program.

    - Dimensions that must conform to the PQC, Non-6σ Program are listed below.  Inspection requirements for these dimensions will be agreed upon by the GM DRE, supplier, GM SQE, and GM technical specialist.

      - None

    - Dimensions that must conform to the AQC, Non-6σ Program are listed below.  Inspection requirements for these dimensions will be agreed upon by the GM DRE, supplier, GM SQE, and GM technical specialist.

      - Leak Testing requirements

    - Dimensions that must conform to DR requirements are listed below:

      - Cylinder block mounting surface flatness (between bolts)

      - Cylinder block mounting surface finish

      - Transmission mounting face profile tolerance

      - RTV step chamfer height

      - Drain plug threaded hole perpendicularity

      - Dowel pin position and perpendicularity tolerance

      - Part cleanliness requirement

  - Part Marking, Labelling and Identification Requirements

    - Parts, components, assemblies and modules identified as requiring Traceability through the Key Characteristics Designation System (KCDS) shall be encoded in a Data Matrix. These parts shall have the Direct Marked Part zone identified on the part drawing / UG math. Consistent marking placement is essential when fixed mount scanners are used at the point of scan

    - The requirements for trace information shall include:

      - Direct part marking per GMW15862.

      - This pan must use a 2D matrix for error-proofing, in accordance with the GMW 15862 Revised June 2013 and SP-E-DPM Version G2.0 specification.  Additional information is also provided on the assembly part drawing.

    - No cast-in GM assembly part numbers are to be used, unless otherwise specified.  If the supplier needs to add an identifier, they must have the GM SQE and DRE approval in writing.

    - During Production, the final assemblies must be marked in a permanent manner with a serial number after successful completion of leak testing in the same station and prior to releasing the assembly from the station.

    - The serial number must be traceable to all in-process quality checks such as, but not limited to, the following parameters:

      - Leak test results

      - Fastener torque values

      - Press-in force values

    - A laser etching marking system must be used for marking parts with the 2D matrix traceability information.

    - The date of manufacture (DOM) must be marked on Production assemblies in an easily viewed location approved by the GM SQE and DRE.

    - The DOM is to include the following information in a format, size and location agreed upon by the GM SQE and DRE:

      - Month, day, year

      - Shift

      - Tool / cavity number

      - Production line

    - The GM part number for the assembly must be marked on Production parts unless otherwise requested and approved by the GM SQE and DRE.  However, the supplier must specify if there is an additional cost for marking each assembly with the GM part number during Production.

      - Part identification will be managed as standard practice during Production via labeled shipping (dunnage) containers.

      - The GM SQE and DRE must approve the method of marking parts with a part number.

    - All Experimental and Prototype parts must be marked with a GM part number including suffix and release level unless otherwise approved by the GM SQE and DRE.

      - The supplier may propose a method of marking the parts for approval by the GM SQE and DRE. A few examples of methods used to mark experimental and prototype parts include, but are not limited to:

        - Laser Etching

        - Dot matrix indent marking system

        - Scribing

        - Application of labels.

    - The following sources detail various methods for marking parts with a part number:

      - www.columbiamt.com

      - www.technifor.com

    - Experimental, Prototype, and Production part assemblies may be marked with the supplier’s manufacturing facility identification number if approved by the GM SQE and DRE.

    - Probable features may be required on the component for error proofing or identification.  Size, shape, and location must be approved by the GM SQE and DRE.

    - The part assembly must not be marked with a supplier logo unless otherwise approved by the GM SQE and DRE.

    - The GM logo must not be marked on the part unless otherwise requested and approved by the GM DRE.

    - The part assembly must not be marked with country of origin unless otherwise approved by the GM SQE and DRE.

    - Certain countries may have emissions certification requirements that require parts to be marked with a supplier or part identification label.

      - Re-certification may be required if markings change during a product life cycle.

      - It is recommended to define a part identification label that is not associated with the part number.  This strategy avoids unnecessary re-certification requirements.

# Performance Requirements

## Warranty:

## The IPTV (Incidents per Thousand Vehicles) warranty is of critical importance.  The design objective is to reduce IPTV by 25% in comparison to the previous model year engine program.

    - The IPTV target for this assembly is a maximum value of 0.1 in any given one year period.

## Root Cause Analysis:

## The supplier is responsible for analyzing all failed parts to determine the root cause of each failure regardless of if the part warranty meets the IPTV target.

## The supplier must create and maintain a Pareto chart to summarize the history of part failures and defects.  The Pareto chart must be created / updated within 20 working days after receipt of a part and/or dealer repair order (verbatims).

## Failure Incident Reports:

## A part failure incident report must be created for the top 3 failure modes.

    - The supplier is required to visit dealers and inspect engines and parts as necessary to determine the root cause of warranty claims.

    - A copy of the part failure incident report must be submitted to the GM DRE and Technical Specialist (TS).

# Analysis, Development and Validation (ADV) Testing Requirements

  - ADV Planning:

    - ADV Plan responsibilities are defined in Appendix A.

    - The supplier must review the ADV test plan with the GM DRE for approval and to define those tests which the Supplier will be responsible for completing.

    - The supplier must complete the required component analysis, development, and validation testing as listed in the ADV plan in accordance with Program Plan timing.

    - The supplier is to prepare a Design Review for presentation to the GM DRE upon completion of the ADV test plan.  The Design Review must be based on test results (DRBTR – design review based on test results) and address all DFMEA items to ensure the part meets or exceeds all design requirements.

    - The assemblies must successfully pass the GM approved ADV test plan and engine validation process to demonstrate design robustness.

    - The GM ADV test plan and engine validation process includes component (stationary fixture), engine dynamometer, and vehicle based testing.

  - Analysis

    - Computational analysis requirements must include, but are not limited to, the following:

      - The supplier is responsible for completing material mold-flow and thermal cooling analyses.  Both types of analysis will be performed to help identify areas of concern for casting, machining, warping, shrinkage, and/or porosity.  The results of this analysis are to be reviewed by GM SQE and DRE prior to constructing the tool and prior to any proposed tool modification.  “Magmasoft ®” (www.magmasoft.com) or an equivalent analysis program must be used to evaluate the tool design.

      - The supplier is responsible for completing a stamped part formability analysis.  Analysis must be performed by the supplier and reviewed by the GM SQE and DRE prior to die construction and prior to any die modification proposed by the customer or supplier.  “Autoform” or an equivalent analysis program must be used to evaluate the tool design.

      - The supplier must analyze design model to identify any thin wall / thick wall conditions, draft angles, and insert sizing / angle / pull direction issues that may affect part forming.

      - The supplier is to submit a written report to the GM DRE detailing the analysis results, including a complete data set for material properties.

## Validation Testing:

    - Validation tests required to be completed by the supplier are located in Appendix G.

    - All records, reports, data and other documentation are to be provided to the GM DRE in electronic format.

# supporting information

## Casting Requirements for Aluminum Components

    - GM Customer Specifics and ISO/IATF 16949 require all suppliers globally to be in compliance with AIAG CQI-27 Casting System Assessment document.  Applicability and effectiveness of casting processes shall be determined utilizing CQI-27 Special Process: Casting System Assessment, published by AIAG, and records maintained.  Supplier shall also meet the minimum requirements in separately supplied Appendix F ‘Other Requirements’ Process SOR CG5305 for Aluminum High Pressure Die Cast Process

    - The supplier must be able to produce the part without having to request changes which would require a 3D math model change.  If unable to produce the part according to the supplied math model and part prints, the supplier must bring this to attention in their quote and at the technical review.

    - Some details, such as clamping features and parting lines must be incorporated based on both GM and Supplier input.

      - Casting and machining locators may be added by the Supplier but must fit within given packaging constraints and be approved by the GM DRE.

      - The supplier is expected to provide expertise for the location of clamp pads, draft angles, blend radii, wall thicknesses, and overall part geometry based on manufacturing guidelines and constraints.

    - The supplier is responsible for completing material flow and thermal analyses.

      - Both analyses must be reviewed by the GM DRE and SQE prior to die construction and prior to implementing any die modification proposal.  The Magmasoft ® analysis program (www.magmasoft.com) or equivalent must be used to evaluate the casting design.

    - The cast part must be free of cracks, inclusions, chills and/or any other defects detrimental to part performance.

    - The cast part must be free from burrs, sharp corners, and rough edges that might be detrimental to satisfactory assembly, safe handling and/or part function.

    - In areas where gating and flash are removed from the casting, adjacent to the sealing surfaces, break-ins must be less than 2 mm.  Visible folds or cold flow lines longer than 3 mm on the machined sealing surfaces are not acceptable.

    - All cast surfaces are to have a 1.0 mm profile tolerance in the clamped state unless specified otherwise.

  - Porosity Requirements and Guidelines.

    - Casting porosity must be controlled on all machined surfaces. Any casting defects that cause oil leaks through the casting with oil dye present is the tier-1 supplier’s responsibility to resolve and must be documented through the five-phase process (see Section 13.0).

    - Porosity as defined here includes inclusions, gas, and shrink types of porosity.  Pores are measured at the maximum diameter or length by maximum depth.  Pores must be spaced a minimum of 3 mm apart.  Pores closer than 3 mm are considered as a single pore (exception: salt and pepper porosity).  The size, frequency, and distribution of porosity on all surfaces shall meet the requirements detailed below:

      - Flanges for RTV sealant: Non-connecting pin holes between 0.0 mm diameter to 0.25 mm diameter (salt and pepper) are permitted, with no limit on the number of occurrences.  Non-connecting pores of <0.5 mm by 0.5 mm are permitted on the seal surface, with minimum spacing of 3.0 mm.  No limit on the number of occurrences.

      - Threaded boss surface:  Non-connecting pores of <2.0 mm by 0.5 mm are permitted on the spot face as long as the pore does not break into the drilled hole.  Pores must be separated by 4 mm and the maximum number of occurrences is 3 per boss.

      - Threaded holes:  Non-connecting pores of <0.5mm by 0.5 are permitted in the first three threads, with a maximum of two occurrences permitted.  The remaining threads may have pores 2 mm in diameter by 2 mm deep, and the pores must be separated by 4 mm and the maximum number of occurrences is 3 per hole.

      - All other machined surfaces:  Non-connecting pin holes between 0.0 mm diameter to 0.25 mm diameter (salt and pepper) are permitted, with no limit on the number of occurrences.  Non-connecting pores of <2 mm by 2 mm deep are permitted, with a minimum of 50 mm between pores.

## Impregnation Requirements:

    - Impregnation of parts is not permitted without written approval from the GM SQE and DRE.

  - Assembly Requirements

    - Assembly dimensional requirements, torques and leak test specifications are included in the document below:

  - Machining Requirements:

    - The part must be free from burrs, sharp corners, and rough edges that might be detrimental to satisfactory assembly, safe handling and/or part function. Special deburring operations may be required. Intersecting holes in high pressure galleries must be brushed (with non-ferrous bristles) to remove burrs.

    - Machining tolerances and surface finish requirements are included in the document below:

    - NOTE: The machined casting must be leak tested individually, prior to installing any other components.

  - Stamping Requirements:

    - Stampings shall be free of cracks, thinning of material, stress risers or sharp corners that may be lead to part failure in its usage.

    - The stamped part must be free from burrs, sharp corners, and rough edges that might be detrimental to satisfactory assembly, safe handling or function of part.

    - Dimensional tolerances and surface finish requirements are included in the document below:

Approximate mass of aluminum die cast oil pan(w/machined):2510g

# Revision History

| Date | Paragraph | Comment |
| --- | --- | --- |
| 26Jun2024 |  |  |