eSOR #204090

#### Appendix-E:

#### Manufacturing Requirements

Engine Variant:		SGE -LSE

Component Name:	Oil Pan Assembly

Date:   				09JULY2024

Revision: 			Version 2.0

Manufacturing Requirements for Purchased Parts

The supplier is responsible for ensuring that the supplied commodity complies with the GM SOR Manufacturing Requirements.  Any anticipated SOR Manufacturing Requirement incompatibilities must be accompanied with an approved deviation using the GM E-dev process, prior to sourcing.

Engine Assembly & Test Bill of Process (BOP) - Manufacturing Requirements for ‘oil pan asm’

- Must be able to install oil pan to the engine with a single, ‘perpendicular to sealing flange’ motion.

- Part identification and part type error-proofing must be achieved by use of a scan-able feature (i.e. 2D matrix) for proliferated oil pan assemblies where a ‘mis-build’ is possible.  Refer to Central Mfg Responsible Engineer (MRE) for further info.  Supplier shall include 2D matrix plus human readable character string to be applied with a laser either to a label that is affixed to the oil pan OR a direct-part-marking on the oil pan (preferred).

  - Reference GM Specification (GMW15862) for both 2D marking (machine readable) and 22 digit PUN (Part Unique Number; human readable) requirement

  - Reference SP-E DPM Direct Part Marking and Validation Specification

  - Readability of barcode to be validated at Pontiac PPO vision lab

  - CG2503 Bar Code Label Content and Format Validation Form shall be completed and submitted prior to PPAP

  - 2D matrix location shall be dimensioned on part print relative to reference feature that can be physically measured to.  This will enable automatic scanners to be set up at engine assembly plant.  Reference template drwg AVT02507

  - 2D matrix location must be common across engine families built at same location

  - 2D matrix marking must be common across engine families built at same location (no mix of labels, direct ink jet & direct laser marking allowed)

Pre-assembled content must include the following:

  - Oil pan

  - Compression limiters (if design requires)

  - Captured fasteners, fully seated or offset to a specific ‘staging’ height

    - This MR will be relaxed IF a common fastener part number is used for all of the lower oil pan mounting fasteners AND there is no possibility of an operator dropping a fastener into the engine during oil pan assembly.

    - IF there are some fasteners more at risk of being dropped into the engine during pan assembly to block, these bolts must be considered to be ‘captured’ (i.e. POA).

    - Any fasteners that are captured (i.e. POA) must remain vertical when oil pan is placed on the engine.

    - NOTE: Even if captured fasteners are NOT included in the design, the supplier is asked to provide an OPTIONAL QUOTATION including captured fasteners.

  - Drain plug

  - Sensors (if design requires)

  - Seals / Gaskets (if design requires and not RTV)

  - Baffles (if design requires/allows)

  - Oil Level Switch / sensor and respective wiring/connector (if design requires/allows)

  - CAST PANS:  POA dowels (datum B & C) with 2 additional holes (machined) for locating part to RTV fixture

    - Hole geometry is 8mm & machined to cast datum

    - Dowels are to be sunk 6mm to enable locating to RTV fixture pins

  - Oil Pump, deflector tray and other content as specified in Appendix C SOR DK68 114607 / 201011

- Shall be dowelled design only

  - PLASTIC: molded-in alignment pins

  - SHEET METAL: dowel pins pressed into mating component

  - CAST ALUMINUM: dowel pins pressed into oil pan (POA), sunk 6mm

- Oil pan to have unique, dedicated locating features which allow for proper alignment of oil pan to RTV application fixture.

  - PLASTIC: (2) 6mm diameter molded ‘thru-holes’ in the oil pan flange (preferred in a diagonal position on the oil pan flange)

  - SHEET METAL/CAST ALUMINUM: (2) 6mm diameter ‘thru-holes’ in the oil pan flange (preferred in a diagonal position on the oil pan flange).  Hole geometry 8mm & machined to cast datum.

  - RTV fixture location holes (2) shall NOT be within 25mm, center to center, of any POA fastener

- Service and assembly repair requirements:  pan shall include jack screws to enable dis-assembly

- May require pry tabs if part is a very small lower oil pan stamped component

- Processing mfg holes may be required for engine plant assembly processing, ie, robot gripper holes (when applicable) machined to specified GD&T

- Further definition of RTV locating holes/features may be required

- If pan has oil level sensor protruding through pan, a pan feature may be required to protect sensor against damage during shipping

- RTV surface (datum A) shall maintain a flatness of 1.5mm in unclamped state.

- RTV ‘step-chamfer’ to be included in oil pan design.

Fastening

- Proper socket clearance per Engine Assembly & Test BOP, Version 5.0, shall be adhered to for this design.

- Bolt spacing shall be >or= to 45mm to accommodate requirements for multi-spindle powerheads

- POA fastener center to center distance to dowel hole used by RTV fixture locating pin shall be > 25mm

- Oil pan shall have no clamping and/or alignment requirements during fastening.

- Bolt pattern (oil pan to lower crankshaft extension/ or block) shall be common for proliferated oil pans within an engine family.

- Handling tabs/handling feature are required to enable operator to keep fingers away from smearing RTV bead, thus enabling avoidance of quality issues and hence supporting goal of flawless launch.

- Features required to enable use of lift assist if applicable.

- 2 manufacturing holes on transmission face and 1 hole on front face to enable part handing in processing station. (GD&T of said holes to follow MRE to DRE)

- Any other features required for automatic handling in process station (engine plant)

- Any POA harnesses retained out of path of RTV & locating areas

- Oil level sensor straight line access must be provided for autostation probe verification

- Part print RTV hole assigned Hole ID#, identified as “RTV process hole”, and GD&T call out as agreed to by MRE

BOSE (GPS ME Bill of Sediment Elimination)

Please refer to chart below

Appendix E SOR Program

Change Log

This Change Log lists all approved changes implemented in this document since the last published version.  For the original release of the document, the Change Log is empty.

| Paragraph Affected | Description of Change | Approver / Approval Date | Requester | Changes Included in Version |
| --- | --- | --- | --- | --- |
|  | This Appendix E provided for eSOR 31589 (DRE: Ziad Alchammas ; Program: CSS 675T; 4-cyl lower oil pan assembly) |  |  |  |
|  | This Appendix E updated to show it is also being used for the following eSORs: 32506 & 32501 (675T A2xx & 675T A2xx RWD oil pans, respectively. DRE: Ziad Alchammas; Program: CSS 675T) | 21JUL14 | Michael Bossner | Minimum height of molded-in alignment pins added |
|  | This Appendix E updated to show it is also being used for the following eSORs: 33321 (45T L3T 3-cyl lower oil pans. DRE: Ziad Alchammas; Program: CSS 45T) | 03SEP14 | Michael Bossner | No content changes from 21JUL14 version |
|  | This Appendix E updated to show it is also being used for the following eSORs: 33714 (40V 3-cyl lower oil pan. DRE: Giorgio Candela; Program: CSS 40V) | 03OCT14 | Michael Bossner | Additional content relative to address the flatness concerns on a molded plastic oil pan |
|  | This Appendix E updated to show it is also being used for the following eSORs: 34034 (45T 4-cyl lower oil pan; DRE: Ziad Alchammas; Program: CSS 45T 4cyl) | 23OCT14 | Michael Bossner |  |
|  | This Appendix E updated to show it is also being used for the following eSORs: 35928 (45T 3-cyl lower oil pan; DRE: Ziad Alchammas; Program: CSS 45T 3cyl) | 20FEB15 | Michael Bossner | MR for ‘single vertical motion’ changed to ‘single, perpendicular to sealing flange motion’ |
|  | This Appendix E updated to show it is also being used for the following eSORs: 36179 (45T 4-cyl lower oil pan; DRE: Ziad Alchammas; Program: CSS 45T 4cyl; LYQ) | 26FEB15 | Michael Bossner |  |
|  | This Appendix E updated to show it is also being used for the following eSORs: 38710 (50V 3-cyl lower oil pan; DRE: Roberto Spina; Program: CSS 50V 3cyl; LXD) | 24JUL15 | Michael Bossner |  |
|  | This Appendix E updated to show it is also being used for the following eSORs: 41883 (50V 4-cyl lower oil pan; DRE: Vincenzo Cognetta; Program: CSS 50V 4cyl; LSQ) | 28JAN16 | Michael Bossner |  |
|  | This Appendix E updated to show it is also being used for the following eSORs: 43556 (50V 3-cyl lower oil pan; DRE: Sergio Capello; Program: CSS 50V 3cyl; LXK; GEM India) | 09MAY16 | Michael Bossner |  |
|  | This Appendix E updated to show it is also being used for the following eSORs: 46610 (50V 4-cyl lower oil pan; DRE: Giuseppe Larocca; Program:2021 CSS 50V 4cyl; LK2) | 04NOV16 | Mary Schimmel |  |
|  | This Appendix E updated to show it is also being used for the following eSORs: 102482 (oil pan asm; DRE: Mariano Jiminez; Program:2022 CSS 375T; LP5) | 17JULY2017 | Mary Schimmel |  |
|  | This Appendix E updated to show it is also being used for the following eSORs: 102666 (oil pan asm; DRE: Fabio Mazza; Program: CSS 50V4 HPD LK2) | 22AUG2017 | Mary Schimmel |  |
|  | This Appendix E updated to show it is also being used for the following eSORs: 102666 (oil pan asm; DRE: Fabio Mazza; Program: CSS 50V4 HPD ) | 30JAN2018 | Mary Schimmel | Added socket clearance requirement. Updated 2D matrix part print req Deleted phy probable boss for EP |
|  | This Appendix E updated to show it is also being used for the following eSORs: 104902 (oil pan asm; DRE: Mariano Jiminez Program: CSS 375T Plastic Oil Pan ) | 19June018 | Mary Schimmel | ‘-Updated diameter of alignment pins or dowels from 8mm to 6mm minimum ‘-Added RTV hole location requirement ‘-Added service and repair requirements |
|  | This Appendix E updated to show it is also being used for the following eSORs: 104946 (oil pan asm; DRE: Mike Ponic Program: CSS 625T Plastic Oil Pan ) | 29June018 | Mary Schimmel | Add bolt spacing requirement |
|  | This Appendix E updated to show it is also being used for the following eSORs: 106662 (oil pan asm; DRE: Puru Mulay Program: HFV6G2 MY24 ) | 07March 2019 | Mary Schimmel | Add handling tab requirement |
|  | This Appendix E updated to show it is also being used for the following eSORs: 107288 (oil pan asm; DRE: N. Simon Program: Prime MY20+ ) | 07March 2019 | Mary Schimmel | Add handling tab requirement |
|  | This Appendix E updated for being used for the following eSOR: 110023 (oil pan asm; DRE: N. Bond, MY 25 GEN 6 ) | 20FEB2020 | Mary Schimmel | Added mfg requirments for eng line processing Gen 6 |
|  | DRE PETER BOTROS SOR 114841 DK68 RPO A MY27 OIL PAN | 23MAY2022 | MARY SCHIMMEL | POA content updated specific to program, RTV processing (robot gripper holes) |
|  |  | June 9, 2022 | MS | Added BOSE items |
|  | JUSTIN AMBROSE PROJECT AUSTIN UPPR OIL PAN SOR 200634 | 02SEPT2022 | M. SCHIMMEL | Straight line access to oil level sensor if applicable. Part print callouts for RTV holes |
|  | Peter Botros SOR 201011 | 25OCT2022 |  | SOR 201011 |